GN Dash (OEM) Cluster

Boost/Tach ANALOG Dash ONLY

| Bulkhead C1 ANALOG A | Pink/blk | GAGES fuse Pos Feed |
| --- | --- | --- |
| B | White | TACH signal |
| C | Brown | Parking Lights |
| D | Gray | Dimmer |
| E | Blk | Gnd |
| F | Gray/blk | 5V MAP feed |
| G | Lt. Green | MAP signal |
| H | Black | MAP ground |

| 1 | Pink/blk | A | GAGES fuse Pos Feed |
| --- | --- | --- | --- |
| 2 | Tan | B | Oil Pressure Gauge |
| 3 | Brn | C | ALT Indicator |
| 4 | Dk. Green | D | Coolant Temp Gauge |
| 5 | Tan/wht | E | BRAKES Warning Lamp |
| 6 | Yellow/blk | F | Seat Belts Warning Lamp |
| 7 | Lt. Blue | G | LH Turn Indicator |
| 8 | Blk | H | Gnd. |
| 9 | Pink | J | Fuel Level Gauge |
| 10 | Gray | K | Dash Light Dimmer ckt. PCB Adapter Harness Wiring |
| 11 | Blk | L | Gnd |
| 12 | - |  | Not used A; Pink/blk; 12V; U on PCB. B; White; Underhood; TACH. D; Gray; Dimmer; G on PCB. E; Blk; Gnd.; S on PCB. F; Pink/Blk; SES Lt.; 1 wire. G; Brn/wht; SES Lt.; 2 wires. H; Lt. Green; Fuel Gauge; B on PCB. |
| 13 | - |  | Not used |
| 14 | - |  | Not used |
| 15 | Pink/blk | M | GAGES fuse Pos Feed |
| 16 | White | N | Cruise Indicator |
| 17 | Dk. Blue | P | RH Turn Indicator |
| 18 | Lt. Green | R | HI Beam Indicator |

C2 DIGITAL Dash ONLY

Bulkhead C1 DIGITAL

| A |  |  |
| --- | --- | --- |
| B | Pink | Fuel Level Gauge |
| C |  |  |
| D |  |  |
| E |  |  |
| F | Brown | Dim Enable |
| G | Gray | Dash Lt. Dimmer |
| H |  |  |
| J |  |  |
| K | Brown | VSS Signal |
| L |  |  |
| M |  |  |
| N |  |  |
| P | Pink/blk | Pos. 12V Switched |
| R |  |  |
| S | Blk/wht | Gnd |
| T |  |  |
| U | Orange | MEM 12V HOT |

| 1 | Tan | A | Oil Pressure Ind |
| --- | --- | --- | --- |
| 2 | Brown | B | ALT Indicator |
| 3 | Dk. Green | C | Water Temp Ind |
| 4 | Tan/wht | D | BRAKES Warning lamp |
| 5 | Yellow/blk | E | Seat Belts Warning Lamp |
| 6 | Lt. Blue | F | LH Turn Indicator |
| 7 | Blk | G | Common LH/Seatblts/HiBm |
| 8 | Lt. Green | H | HI Beam Indicator lamp |
| 9 | - |  | Not used |
| 10 | Blk | J | Gnd |
| 11 | - | K | Not used |
| 12 | Wht/blk |  | LO Boost Indicator |
| 13 | Blue/yel |  | HI Boost Indicator |
| 14 | Pink/blk | L | Switched 12 V |
| 15 | White | M | Cruise Indicator |
| 16 | Dk. Blue | N | RH Turn Indicator |
| 17 | Gray | P | Dash light dimmer ckt. |
| 18 | Pink/blk | R | Pos. 12 V dash lights |

Boost switch operates at (Economy) 2.5PSI, and (Power) 10PSI. Neither are used on GNX wiring.

Move pink wire in GNX C1 receptacle from 9 to 14. Insert in “R”.

Remove pink “fuel Gauge” wire from cluster harness, add 280F terminal, and place in H for tach signal (white connector).